ChangeUID Code:

/*

* --------------------------------------------------------------------------------------------------------------------

* Example to change UID of changeable MIFARE card.

* --------------------------------------------------------------------------------------------------------------------

* This is a MFRC522 library example; for further details and other examples see: https://github.com/miguelbalboa/rfid

*

* This sample shows how to set the UID on a UID changeable MIFARE card.

*

* @author Tom Clement

* @license Released into the public domain.

*

* Typical pin layout used:

* -----------------------------------------------------------------------------------------

*             MFRC522      Arduino       Arduino   Arduino    Arduino          Arduino

*             Reader/PCD   Uno/101       Mega      Nano v3    Leonardo/Micro   Pro Micro

* Signal      Pin          Pin           Pin       Pin        Pin              Pin

* -----------------------------------------------------------------------------------------

* RST/Reset   RST          9             5         D9         RESET/ICSP-5     RST

* SPI SS      SDA(SS)      10            53        D10        10               10

* SPI MOSI    MOSI         11 / ICSP-4   51        D11        ICSP-4           16

* SPI MISO    MISO         12 / ICSP-1   50        D12        ICSP-1           14

* SPI SCK     SCK          13 / ICSP-3   52        D13        ICSP-3           15

*

* More pin layouts for other boards can be found here: https://github.com/miguelbalboa/rfid#pin-layout

*/

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN   9     // Configurable, see typical pin layout above

#define SS_PIN    10    // Configurable, see typical pin layout above

MFRC522 mfrc522(SS_PIN, RST_PIN);   // Create MFRC522 instance

/* Set your new UID here! */

#define NEW_UID {0xDE, 0xAD, 0xBE, 0xEF}

void setup() {

Serial.begin(9600);  // Initialize serial communications with the PC

while (!Serial);     // Do nothing if no serial port is opened (added for Arduinos based on ATMEGA32U4)

SPI.begin();         // Init SPI bus

mfrc522.PCD_Init();  // Init MFRC522 card

Serial.println(F("Warning: this example overwrites the UID of your UID changeable card, use with care!"));

}

// Setting the UID can be as simple as this:

//void loop() {

//  byte newUid[] = NEW_UID;

//  if ( mfrc522.MIFARE_SetUid(newUid, (byte)4, true) ) {

//    Serial.println("Wrote new UID to card.");

//  }

//  delay(1000);

//}

// But of course this is a more proper approach

void loop() {

// Reset the loop if no new card present on the sensor/reader. This saves the entire process when idle. And if present, select one.

if ( ! mfrc522.PICC_IsNewCardPresent() || ! mfrc522.PICC_ReadCardSerial() ) {

delay(50);

return;

}

// Now a card is selected. The UID and SAK is in mfrc522.uid.

// Dump UID

Serial.print(F("Card UID:"));

for (byte i = 0; i < mfrc522.uid.size; i++) {

Serial.print(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : " ");

Serial.print(mfrc522.uid.uidByte[i], HEX);

}

Serial.println();

// Dump PICC type

//  MFRC522::PICC_Type piccType = mfrc522.PICC_GetType(mfrc522.uid.sak);

//  Serial.print(F("PICC type: "));

//  Serial.print(mfrc522.PICC_GetTypeName(piccType));

//  Serial.print(F(" (SAK "));

//  Serial.print(mfrc522.uid.sak);

//  Serial.print(")\r\n");

//  if (  piccType != MFRC522::PICC_TYPE_MIFARE_MINI

//    &&  piccType != MFRC522::PICC_TYPE_MIFARE_1K

//    &&  piccType != MFRC522::PICC_TYPE_MIFARE_4K) {

//    Serial.println(F("This sample only works with MIFARE Classic cards."));

//    return;

//  }

// Set new UID

byte newUid[] = NEW_UID;

if ( mfrc522.MIFARE_SetUid(newUid, (byte)4, true) ) {

Serial.println(F("Wrote new UID to card."));

}

// Halt PICC and re-select it so DumpToSerial doesn't get confused

mfrc522.PICC_HaltA();

if ( ! mfrc522.PICC_IsNewCardPresent() || ! mfrc522.PICC_ReadCardSerial() ) {

return;

}

// Dump the new memory contents

Serial.println(F("New UID and contents:"));

mfrc522.PICC_DumpToSerial(&(mfrc522.uid));

delay(2000);

}

Dumpinfo RFID Clone:

/*

* --------------------------------------------------------------------------------------------------------------------

* Example sketch/program showing how to read data from a PICC to serial.

* --------------------------------------------------------------------------------------------------------------------

* This is a MFRC522 library example; for further details and other examples see: https://github.com/miguelbalboa/rfid

*

* Example sketch/program showing how to read data from a PICC (that is: a RFID Tag or Card) using a MFRC522 based RFID

* Reader on the Arduino SPI interface.

*

* When the Arduino and the MFRC522 module are connected (see the pin layout below), load this sketch into Arduino IDE

* then verify/compile and upload it. To see the output: use Tools, Serial Monitor of the IDE (hit Ctrl+Shft+M). When

* you present a PICC (that is: a RFID Tag or Card) at reading distance of the MFRC522 Reader/PCD, the serial output

* will show the ID/UID, type and any data blocks it can read. Note: you may see "Timeout in communication" messages

* when removing the PICC from reading distance too early.

*

* If your reader supports it, this sketch/program will read all the PICCs presented (that is: multiple tag reading).

* So if you stack two or more PICCs on top of each other and present them to the reader, it will first output all

* details of the first and then the next PICC. Note that this may take some time as all data blocks are dumped, so

* keep the PICCs at reading distance until complete.

*

* @license Released into the public domain.

*

* Typical pin layout used:

* -----------------------------------------------------------------------------------------

*             MFRC522      Arduino       Arduino   Arduino    Arduino          Arduino

*             Reader/PCD   Uno/101       Mega      Nano v3    Leonardo/Micro   Pro Micro

* Signal      Pin          Pin           Pin       Pin        Pin              Pin

* -----------------------------------------------------------------------------------------

* RST/Reset   RST          9             5         D9         RESET/ICSP-5     RST

* SPI SS      SDA(SS)      10            53        D10        10               10

* SPI MOSI    MOSI         11 / ICSP-4   51        D11        ICSP-4           16

* SPI MISO    MISO         12 / ICSP-1   50        D12        ICSP-1           14

* SPI SCK     SCK          13 / ICSP-3   52        D13        ICSP-3           15

*

* More pin layouts for other boards can be found here: https://github.com/miguelbalboa/rfid#pin-layout

*/

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN         9          // Configurable, see typical pin layout above

#define SS_PIN          10         // Configurable, see typical pin layout above

MFRC522 mfrc522(SS_PIN, RST_PIN);  // Create MFRC522 instance

void setup() {

Serial.begin(9600);     // Initialize serial communications with the PC

while (!Serial);        // Do nothing if no serial port is opened (added for Arduinos based on ATMEGA32U4)

SPI.begin();            // Init SPI bus

mfrc522.PCD_Init();     // Init MFRC522

delay(4);               // Optional delay. Some board do need more time after init to be ready, see Readme

mfrc522.PCD_DumpVersionToSerial();  // Show details of PCD - MFRC522 Card Reader details

Serial.println(F("Scan PICC to see UID, SAK, type, and data blocks..."));

}

void loop() {

// Reset the loop if no new card present on the sensor/reader. This saves the entire process when idle.

if ( ! mfrc522.PICC_IsNewCardPresent()) {

return;

}

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial()) {

return;

}

// Dump debug info about the card; PICC_HaltA() is automatically called

mfrc522.PICC_DumpToSerial(&(mfrc522.uid));

}

RFID Cloner:

/*

* Copy the RFID card data into variables and then

* scan the second empty card to copy all the data

* ----------------------------------------------------------------------------

* Example sketch/program which will try the most used default keys listed in

* https://code.google.com/p/mfcuk/wiki/MifareClassicDefaultKeys to dump the

* block 0 of a MIFARE RFID card using a RFID-RC522 reader.

*

* Typical pin layout used:

* -----------------------------------------------------------------------------------------

*             MFRC522      Arduino       Arduino   Arduino    Arduino          Arduino

*             Reader/PCD   Uno/101       Mega      Nano v3    Leonardo/Micro   Pro Micro

* Signal      Pin          Pin           Pin       Pin        Pin              Pin

* -----------------------------------------------------------------------------------------

* RST/Reset   RST          9             5         D9         RESET/ICSP-5     RST

* SPI SS      SDA(SS)      10            53        D10        10               10

* SPI MOSI    MOSI         11 / ICSP-4   51        D11        ICSP-4           16

* SPI MISO    MISO         12 / ICSP-1   50        D12        ICSP-1           14

* SPI SCK     SCK          13 / ICSP-3   52        D13        ICSP-3           15

*

* More pin layouts for other boards can be found here: https://github.com/miguelbalboa/rfid#pin-layout

*

*/

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN         9           // Configurable, see typical pin layout above

#define SS_PIN          10          // Configurable, see typical pin layout above

MFRC522 mfrc522(SS_PIN, RST_PIN);   // Create MFRC522 instance.

byte buffer[18];

byte block;

byte waarde[64][16];

MFRC522::StatusCode status;

MFRC522::MIFARE_Key key;

// Number of known default keys (hard-coded)

// NOTE: Synchronize the NR_KNOWN_KEYS define with the defaultKeys[] array

#define NR_KNOWN_KEYS   8

// Known keys, see: https://code.google.com/p/mfcuk/wiki/MifareClassicDefaultKeys

byte knownKeys[NR_KNOWN_KEYS][MFRC522::MF_KEY_SIZE] =  {

{0xff, 0xff, 0xff, 0xff, 0xff, 0xff}, // FF FF FF FF FF FF = factory default

{0xa0, 0xa1, 0xa2, 0xa3, 0xa4, 0xa5}, // A0 A1 A2 A3 A4 A5

{0xb0, 0xb1, 0xb2, 0xb3, 0xb4, 0xb5}, // B0 B1 B2 B3 B4 B5

{0x4d, 0x3a, 0x99, 0xc3, 0x51, 0xdd}, // 4D 3A 99 C3 51 DD

{0x1a, 0x98, 0x2c, 0x7e, 0x45, 0x9a}, // 1A 98 2C 7E 45 9A

{0xd3, 0xf7, 0xd3, 0xf7, 0xd3, 0xf7}, // D3 F7 D3 F7 D3 F7

{0xaa, 0xbb, 0xcc, 0xdd, 0xee, 0xff}, // AA BB CC DD EE FF

{0x00, 0x00, 0x00, 0x00, 0x00, 0x00}  // 00 00 00 00 00 00

};

char choice;

/*

* Initialize.

*/

void setup() {

Serial.begin(9600);         // Initialize serial communications with the PC

while (!Serial);            // Do nothing if no serial port is opened (added for Arduinos based on ATMEGA32U4)

SPI.begin();                // Init SPI bus

mfrc522.PCD_Init();         // Init MFRC522 card

Serial.println(F("Try the most used default keys to print block 0 to 63 of a MIFARE PICC."));

Serial.println("1.Read card \n2.Write to card \n3.Copy the data.");

for (byte i = 0; i < 6; i++) {

key.keyByte[i] = 0xFF;

}

}

//Via seriele monitor de bytes uitlezen in hexadecimaal

void dump_byte_array(byte *buffer, byte bufferSize) {

for (byte i = 0; i < bufferSize; i++) {

Serial.print(buffer[i] < 0x10 ? " 0" : " ");

Serial.print(buffer[i], HEX);

}

}

//Via seriele monitor de bytes uitlezen in ASCI

void dump_byte_array1(byte *buffer, byte bufferSize) {

for (byte i = 0; i < bufferSize; i++) {

Serial.print(buffer[i] < 0x10 ? " 0" : " ");

Serial.write(buffer[i]);

}

}

/*

* Try using the PICC (the tag/card) with the given key to access block 0 to 63.

* On success, it will show the key details, and dump the block data on Serial.

*

* @return true when the given key worked, false otherwise.

*/

bool try_key(MFRC522::MIFARE_Key *key)

{

bool result = false;

for(byte block = 0; block < 64; block++){

// Serial.println(F("Authenticating using key A..."));

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return false;

}

// Read block

byte byteCount = sizeof(buffer);

status = mfrc522.MIFARE_Read(block, buffer, &byteCount);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Read() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

}

else {

// Successful read

result = true;

Serial.print(F("Success with key:"));

dump_byte_array((*key).keyByte, MFRC522::MF_KEY_SIZE);

Serial.println();

// Dump block data

Serial.print(F("Block ")); Serial.print(block); Serial.print(F(":"));

dump_byte_array1(buffer, 16); //omzetten van hex naar ASCI

Serial.println();

for (int p = 0; p < 16; p++) //De 16 bits uit de block uitlezen

{

waarde [block][p] = buffer[p];

Serial.print(waarde[block][p]);

Serial.print(" ");

}

}

}

Serial.println();

Serial.println("1.Read card \n2.Write to card \n3.Copy the data.");

mfrc522.PICC_HaltA();       // Halt PICC

mfrc522.PCD_StopCrypto1();  // Stop encryption on PCD

return result;

start();

}

/*

* Main loop.

*/

void loop() {

start();

}

void start(){

choice = Serial.read();

if(choice == '1')

{

Serial.println("Read the card");

keuze1();

}

else if(choice == '2')

{

Serial.println("See what is in the variables");

keuze2();

}

else if(choice == '3')

{

Serial.println("Copying the data on to the new card");

keuze3();

}

}

void keuze2(){ //Test waardes in blokken

for(block = 4; block <= 62; block++){

if(block == 7 || block == 11 || block == 15 || block == 19 || block == 23 || block == 27 || block == 31 || block == 35 || block == 39 || block == 43 || block == 47 || block == 51 || block == 55 || block == 59){

block ++;

}

Serial.print(F("Writing data into block "));

Serial.print(block);

Serial.println("\n");

for(int j = 0; j < 16; j++){

Serial.print(waarde[block][j]);

Serial.print(" ");

}

Serial.println("\n");

}

Serial.println("1.Read card \n2.Write to card \n3.Copy the data.");

start();

}

void keuze3(){ //Copy the data in the new card

Serial.println("Insert new card...");

// Look for new cards

if ( ! mfrc522.PICC_IsNewCardPresent())

return;

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial())

return;

// Show some details of the PICC (that is: the tag/card)

Serial.print(F("Card UID:"));

dump_byte_array(mfrc522.uid.uidByte, mfrc522.uid.size);

Serial.println();

Serial.print(F("PICC type: "));

MFRC522::PICC_Type piccType = mfrc522.PICC_GetType(mfrc522.uid.sak);

Serial.println(mfrc522.PICC_GetTypeName(piccType));

// Try the known default keys

/*MFRC522::MIFARE_Key key;

for (byte k = 0; k < NR_KNOWN_KEYS; k++) {

// Copy the known key into the MIFARE_Key structure

for (byte i = 0; i < MFRC522::MF_KEY_SIZE; i++) {

key.keyByte[i] = knownKeys[k][i];

}

}*/

for (byte i = 0; i < 6; i++) {

key.keyByte[i] = 0xFF;

}

for(int i = 4; i <= 62; i++){ //De blocken 4 tot 62 kopieren, behalve al deze onderstaande blocken (omdat deze de authenticatie blokken zijn)

if(i == 7 || i == 11 || i == 15 || i == 19 || i == 23 || i == 27 || i == 31 || i == 35 || i == 39 || i == 43 || i == 47 || i == 51 || i == 55 || i == 59){

i++;

}

block = i;

// Authenticate using key A

Serial.println(F("Authenticating using key A..."));

status = (MFRC522::StatusCode) mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

// Authenticate using key B

Serial.println(F("Authenticating again using key B..."));

status = (MFRC522::StatusCode) mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_B, block, &key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

// Write data to the block

Serial.print(F("Writing data into block "));

Serial.print(block);

Serial.println("\n");

dump_byte_array(waarde[block], 16);

status = (MFRC522::StatusCode) mfrc522.MIFARE_Write(block, waarde[block], 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

}

Serial.println("\n");

}

mfrc522.PICC_HaltA();       // Halt PICC

mfrc522.PCD_StopCrypto1();  // Stop encryption on PCD

Serial.println("1.Read card \n2.Write to card \n3.Copy the data.");

start();

}

void keuze1(){ //Read card

Serial.println("Insert card...");

// Look for new cards

if ( ! mfrc522.PICC_IsNewCardPresent())

return;

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial())

return;

// Show some details of the PICC (that is: the tag/card)

Serial.print(F("Card UID:"));

dump_byte_array(mfrc522.uid.uidByte, mfrc522.uid.size);

Serial.println();

Serial.print(F("PICC type: "));

MFRC522::PICC_Type piccType = mfrc522.PICC_GetType(mfrc522.uid.sak);

Serial.println(mfrc522.PICC_GetTypeName(piccType));

// Try the known default keys

MFRC522::MIFARE_Key key;

for (byte k = 0; k < NR_KNOWN_KEYS; k++) {

// Copy the known key into the MIFARE_Key structure

for (byte i = 0; i < MFRC522::MF_KEY_SIZE; i++) {

key.keyByte[i] = knownKeys[k][i];

}

// Try the key

if (try_key(&key)) {

// Found and reported on the key and block,

// no need to try other keys for this PICC

break;

}

}

}